Origins and purpose

fair v2.1.3 (Leach et al., 2021; Smith, 2023, 2024a) has been incorporated as a module in then TIM-LEAF framework.

The approach of the Climate Change Commission (CCC) in New Zealand was replicated to assess the warming impact of national greenhouse gas emissions pathways (Climate Change Commission, 2024b, 2024a; Smith, 2024b; Smith et al., 2021). This approach uses a modified locally compiled version of the fair v2.1.3 (fair.py) source code so that global baseline emissions scenarios can represent a country level contribution, in this case Ireland’s contribution (Climate Change Commission, 2024b). The modified fair.py has two additional functions (i.e. def fill_scenarios_from_rcmip (self, RCMIPscenario) and def fill_scenario_from_rcmip (self, scenario, RCMIPscenario)), otherwise it is the same as the original.

The first function fills all scenarios that fair will run with the same global reduced complexity model intercomparison project (RCMIP) emissions scenario, in this case Shared Socioeconomic Pathway (SSP) 1-2.6 (Nicholls et al., 2020).

The second function fills a specified fair scenario with a given emissions scenario.

This allows (1) the warming that would have happened without Ireland’s emissions and (2) the difference between the global baseline and the global baseline without Ireland’s emissions, which can be interpreted as the warming caused by Ireland’s emissions’, to be calculated (Climate Change Commission, 2024b).

## Instructions to compile fair with a modified fair.py file

Needs python and pip installed.

This modelling is based on fair v2.1.3. The latest version is fair v2.2.4. There is no change to the physical climate outcome in any way between these two versions as this depends on the calibration dataset.

The fair v2.1.3 source code was modified to

- make it easier to read in a global baseline emissions scenario and

- reflect a country-level contribution (this is no longer necessary as later versions of fair read in emissions in a different way)

To run this version fair will need to be compiled locally with a modified fair.py rather than the standard version that come via python pip.

To do this

- download fair v2.1.3 source code

https://github.com/OMS-NetZero/FAIR/releases under November 23 2023 fair v2.1.3

- replace fair.py (in /FAIR-2.1.3/src/fair/fair.py) with this version fair.py [link to file] (originally sourced from the CCC in New Zealand)

- run setup using "pip install -e ."

The fair.py file is the same as the original, but with two added functions:

def fill_scenarios_from_rcmip(self, RCMIPscenario)

def fill_scenario_from_rcmip(self, scenario, RCMIPscenario)

The first function fills all scenarios that fair will run with the same global RCMIP emissions scenario.

The second function fills a specified fair scenario with a given RCMIP emissions scenario.

## Instructions to run fair for country-level GHG emissions pathways

The Python script countrylevel_IRL.py [link to this file in fair/] used to run this analysis for Ireland, is based on a Jupyter Notebook script [check hyperlink works once on github] from the CCC (Climate Change Commission, 2024a, 2024b). It uses the first of the two added functions and then subtracts the values of Ireland’s emissions (e.g. CO2, N2O and CH4) from global emissions to calculate the warming that would have happened without Ireland’s emissions and the difference between the global baseline and the global baseline minus Ireland’s emissions, is then interpreted as 'the warming caused by Ireland’s emissions'.

The original script was reviewed by Chis Smith for the New Zealand CCC.

Ireland-specific modification: countrylevel_IRL.py was minimally modified to include exchange_data(IRL_scenarios, IRL_scenarios_file), which transfers the Ireland scenario data generated by the TIMLEAF data-transfer script into the corresponding emission template file (IRL_histWheatley2021_<scenario>.csv).

-if someone wants to apply new scenarios to the model, they should apply their data in the same format as BAU.. 
When running the model, changing the IRL_scenario = to the one needed. Add scenario name and scenario path to the IRL_scenarios list.

IRL_scenario = "S3" # BAU, TN, SGLA, SGHA & NZ; the script is run for each of Ireland's emissions scenario individually

IRL_scenarios = [("BAU", 'data/BAU.vd'), ('S1','data/S1.vd'), ('S2', 'data/S2.vd'), ('S3', 'data/S3.vd')]

[Neha, can you describe the modifications that you and Zahra made to this file]

## Inputs

Input files can be found here [link to TIM-LEAF-FaIR_framework/fair/input]

### Calibration

It is important to use the same input files for climate calibration and species properties calibration

- file v1.4.2; calibrated_constrained_parameters-v1.4.2.csv (Smith 2024a, 2024b)

- file v1.2.0; species_configs_properties_calibration1.2.0.csv (Smith 2024a, 2024b)

Note that both have newer versions available, but it is not possible to guarantee that they work with this version of fair (v2.1.3) and this python script. Any difference in actual modelled warming should be so minor as to be not relevant.

Use

- solar_erf_timebounds.csv  (Smith 2024a, 2024b)

- volcanic_ERF_1750-2101_timebounds.csv (Smith 2024a, 2024b)

### Global emissions files

Use

- rcmip-concentrations-annual-means-v5-1-0.csv (Nicholls & Lewis 2021)

- rcmip-emissions-annual-means-v5-1-0.csv (Nicholls & Lewis 2021)

- rcmip-radiative-forcing-annual-means-v5-1-0.csv (Nicholls & Lewis 2021)

### Country-level GHG emissions pathways

A file of combined historical and TIM-LEAF scenarios data e.g.  IRL_histWheatley2021_S3.csv [link to TIM-LEAF-FaIR_framework/data/combined ] is an emissions input file for Ireland (1750 to 2070). Created as detailed here [link to TIM-LEAF to FaIR data transfer / README].

The scenario can have any name but it must match the scenario name provided in the python script. Data are expected annually from (1750 or) 1850 to 2299.

## Key steps

The python countrylevel_IRL.py script has various comments that explain the key steps.

Key steps are to define the global emissions scenario (this is important as the radiative efficacy of a country’s emissions depends on global background concentrations), the name of the specific country-level emissions scenario you want to run (e.g. ‘BAU'), and a historical reference year (e.g. 1990 or 2026).

The historical reference year is used to quantify how much emissions since a given date have contributed to global warming (this is important to show how much of the total warming is due to recent emissions, i.e. CH4, compared to historical emissions).

Now the specific fair scenarios that you want to run are defined (1) the global baseline and then (2) the warming from each individual gas at country level as specified.

There are three different types of scenario

- global emissions scenario (e.g. ssp126)

- Ireland’s emissions scenario (e.g. BAU)

- fair scenarios (e.g. IRLCO2, which is defined as global emissions minus in this case Ireland’s (i.e. country level) fossil fuel CO2 emissions).

Fair 'runs' the fair scenarios. The other scenarios are used only to define the emissions used in those fair scenarios.

After these initial definitions (first block of code) fair is being set up in the standard way.

Then after the line

df_IRL_emis = pd.read_csv(IRL_scenarios_file)

comes a block of code where the calculations are done to define the fair scenarios, i.e. global emissions minus the Ireland emissions of a specific gas; and for the historical reference year, global emissions minus Ireland emissions of an Ireland emissions of a specific gas; and for the historical reference year, global emissions minus Ireland emissions of a specific gas up to the historical reference year.

Now fair is run.

## Outputs

Outputs are written to a file, including various ways of expressing the warming from multiple gases, including the warming from emissions after the historical reference year.

Output appears in a .csv file with a name that is COUNTRY_GLOBALSCENARIO_COUNTRY-SCENARIO&YEAR.csv e.g. IRL_ssp126_BAU2020.csv [link to example in TIM-LEAF-FaIR_workgflow/fair/output]

YEAR is the year you are interested in assessing temperature change from. It can be any year (e.g. 1990, 2015, 2020 or most recent year) depending on what you want from your scenarios analysis.

A file e.g. NZ_ssp126_EB41990.xlsx [link to example in TIM-LEAF-FaIR_workgflow/fair/output (sourced from the CCC in New Zealand)) with some rudimentary graphs to demonstrate how the individual calculated columns can work to show total warming, or the warming from emissions since the historical reference year, i.e. 2020, or the contribution from emissions up to 2020 and from 2021.

## References

​Climate Change Commission. (2024a). Input data files for temperature modelling, New Zealand.

​Climate Change Commission. (2024b). Technical annex final report on the fourth emissions budget and 2050 target review.

​Leach NJ, Jenkins S, Nicholls Z, Smith CJ, Lynch J, Cain M, Walsh T, Wu B, Tsutsui J & Allen MR (2021). FaIRv2.0.0: a generalized impulse response model for climate uncertainty and future scenario exploration. Geoscientific Model Development, 14(5), 3007–3036. https://doi.org/10.5194/gmd-14-3007-2021

Nicholls Z, Meinshausen M, Lewis J, Gieseke R, Dommenget D, Dorheim K, Fan C-S, Fuglestvedt JS, Gasser T, Golüke U, Goodwin P, Hartin C, Hope AP, Kriegler E, Leach NJ, Marchegiani D, McBride LA, Quilcaille Y, Rogelj J, Salawitch RJ, Samset BH, Sandstad M, Shiklomanov AN, Skeie RB, Smith CJ, Smith S, Tanaka K, Tsutsui J & Xie Z (2020). Reduced Complexity Model Intercomparison Project Phase 1: Introduction and evaluation of global-mean temperature response. Geoscientific Model Development, https://doi.org/10.5194/gmd-13-5175-2020

Nicholls Z & Lewis J (2021). Reduced Complexity Model Intercomparison Project (RCMIP) protocol (Version v5.1.0) [Dataset]. Zenodo. https://doi.org/10.5281/zenodo.4589756

​Smith C (2023). FaIR Version 2.1.3. https://doi.org/https://github.com/OMS-NetZero/FAIR/releases

​Smith C (2024a). FaIR calibration data. https://doi.org/https://doi.org/10.5281/zenodo.10566813

​Smith C (2024b). FaIR calibration data (1.4.2) [Dataset]. Zenodo. https://doi.org/https://doi.org/10.5281/zenodo.13142999

Smith C, Gasser T, Nicholls Z, Armour K, Collins W, Forster P, Meinshausen M, Watanabe M (2021). The Earth’s Energy Budget, Climate Feedbacks and Climate Sensitivity Supplementary Material. In Climate Change 2021: The Physical Science Basis. Contribution of Working Group I to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change.  [Masson-Delmotte V, Zhai P, Pirani A,  Connors SL, Péan C, Berger S, Caud N, Chen Y, Goldfarb L, Gomis MI, Huang M, Leitzell, K, Lonnoy E, Matthews JBR, Maycock TK, Waterfield T, Yelekçi O, Yu R & Zhou B (eds.)]

Wheatley, J. (2021). Ireland greenhouse gas and sulphate aerosol emissions 1745-2019. https://doi.org/https://doi.org/10.5281/zenodo.7004406

Wheatley, J. (2023). Temperature neutrality and Irish methane policy. Climate Policy, 23(10), 1229–1242. https://doi.org/10.1080/14693062.2023.2191921